- Test Dataset: 10100AA158005

S‐158 Recommended ENC Validation Checks covered in this section:

| S‐158 Check | Description |
| --- | --- |
| 101_1318 | For each DistanceMark OR FogSignal OR LightAllAround OR LightFogDetector OR PhysicalAISAidToNavigation OR RadarTransponderBeacon OR Retroreflector OR SignalStationTraffic OR SignalStationWarning which is COVERED_BY a DepthArea, DredgedArea or UnsurveyedArea AND does not have a StructureEquipment feature association with a navigation aid structure AND the geometry of which is not COVERED_BY a SpanFixed, SpanOpening, PylonBridgeSupport, CableOverhead, Coastline, Conveyor, Dam with categoryOfDam Equal to 3 (Flood Barrage), LandArea, PipelineOverhead, Pontoon or ShorelineConstruction. |
| 101_1050 | If the DataCoverage is Not equal to the combined coverage of NavigationalSystemOfMarks meta features. |
| 101_1295 | For each DeepWaterRouteCentreline where orientationValue is Known AND trafficFlow is Equal to 1 (inbound) OR 2 (outbound) OR 3 (one-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value of orientationValue. |
| 101_1296 | For each DeepWaterRouteCentreline where orientationValue is Known AND trafficFlow is Equal to 4 (two-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value (or reciprocal value) of orientationValue. |
| 101_1055 | For each NavigationalSystemOfMarks which OVERLAPS or is WITHIN another meta NavigationalSystemOfMarks. |
| 101_1293 | For each RecommendedRouteCentreline where trafficFlow is Equal to 1 (inbound) OR 2 (outbound) OR 3 (one-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value of orientationValue. |
| 101_1294 | For each RecommendedRouteCentreline where orientationValue is Known AND trafficFlow is Equal to 4 (two-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value (or reciprocal value) of orientationValue. |
| 101_1287 | For each RecommendedTrack where orientationValue is Known AND trafficFlow is Equal to 4 (two-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value (or reciprocal value) of orientationValue. |
| 101_1140 | For each Building, BuiltUpArea, Crane, FortifiedStructure, Landmark, SiloTank or WindTurbine without inTheWater set to True which OVERLAPS OR is COVERED_BY DepthArea, DredgedArea or an UnsurveyedArea. |

Secondary Errors

Critical – None

Error – 3

Warnings – 6

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1318 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each DistanceMark OR FogSignal OR LightAllAround OR LightFogDetector OR PhysicalAISAidToNavigation OR RadarTransponderBeacon OR Retroreflector OR SignalStationTraffic OR SignalStationWarning which is COVERED_BY a DepthArea, DredgedArea or UnsurveyedArea AND does not have a StructureEquipment feature association with a navigation aid structure AND the geometry of which is not COVERED_BY a SpanFixed, SpanOpening, PylonBridgeSupport, CableOverhead, Coastline, Conveyor, Dam with categoryOfDam Equal to 3 (Flood Barrage), LandArea, PipelineOverhead, Pontoon or ShorelineConstruction. |  |  |  |  |  |  |  |  |  |
| Message | Navigation aid equipment feature within DepthArea, DredgedArea or UnsurveyedArea without an appropriate supporting structure feature or underlying feature. |  |  |  |  |  |  |  |  |  |
| Solution | Ensure navigational aid equipment feature is associated with an appropriate structure feature or has a suitable underlying feature. |  |  |  | Conformity |  | PS 18.2 |  |  |  |
| Test Case No. 1 | Navigation aid equipment feature within DepthArea, DredgedArea or UnsurveyedArea without an appropriate supporting structure feature or underlying feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
|  | SignalStationTraffic FogSignal FogSignal RadarTransponderBeacon SignalStationWarning Retroreflector RadarTransponderBeacon SignalStationWarning Retroreflector SignalStationTraffic LightAllAround LightAllAround |  |  | 22 33 36 37 38 39 40 41 42 45 52 53 |  | 1810:6875:1 1810:6930:1 1810:6933:1 1810:6934:1 1810:6935:1 1810:6937:1 1810:6939:1 1810:6940:1 1810:6941:1 1810:6946:1 1810:8359:1 1810:8358:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1318: An error “Navigation aid equipment feature within DepthArea, DredgedArea or UnsurveyedArea without an appropriate supporting structure feature or underlying feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1050 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If the DataCoverage is Not equal to the combined coverage of NavigationalSystemOfMarks meta features. |  |  |  |  |  |  |  |  |  |
| Message | DataCoverage not completely covered by NavigationalSystemOfMarks meta features. |  |  |  |  |  |  |  |  |  |
| Solution | Ensure the combined coverage of NavigationalSystemOfMarks features is equal to the combined coverage of DataCoverage features. |  |  |  | Conformity |  | PS 3.5.1 and 3.6.1 |  |  |  |
| Test Case No. 1 | DataCoverage not completely covered by NavigationalSystemOfMarks meta features. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.408485164S, 61.838661714E | DataCoverage | maximumDispalyScale 12000 minimumDisplayScale 180000 optimumDisplayScale 45000 |  | 6 |  | 1810:6845:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1050: An error “DataCoverage not completely covered by NavigationalSystemOfMarks meta features.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1295 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each DeepWaterRouteCentreline where orientationValue is Known AND trafficFlow is Equal to 1 (inbound) OR 2 (outbound) OR 3 (one-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value of orientationValue. |  |  |  |  |  |  |  |  |  |
| Message | orientationValue for DeepWaterRouteCentreline does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Solution | Amend orientationValue to be consistent with orientation of the curve spatial feature it references. |  |  |  | Conformity |  | PS 15.13.1 |  |  |  |
| Test Case No. 1 | orientationValue for DeepWaterRouteCentreline does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.339519716S, 61.772138397E | DeepWaterRouteCentreline | basedOnFixedMarks True orientation/orientationValue 10 trafficFlow 1 |  | 13 |  | 1810:6860:1 |  |  | - |  |  |
| 32.339894057S, 61.771928766E | DeepWaterRouteCentreline | basedOnFixedMarks True orientation/orientationValue 10 trafficFlow 2 |  | 18 |  | 1810:6871:1 |  |  |  |  |  |
| 32.339122914S, 61.772235726E | DeepWaterRouteCentreline | basedOnFixedMarks True orientationValue 10 orientation/orientationValue 3 |  | 21 |  | 1810:6874:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1295: An error “orientationValue for NavigationLine does not correspond to the bearing of the line.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1296 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each DeepWaterRouteCentreline where orientationValue is Known AND trafficFlow is Equal to 4 (two-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value (or reciprocal value) of orientationValue. |  |  |  |  |  |  |  |  |  |
| Message | orientationValue for DeepWaterRouteCentreline does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Solution | Amend orientationValue to be consistent with orientation of the curve spatial feature it references. |  |  |  | Conformity |  | PS 15.13.1 |  |  |  |
| Test Case No. 1 | orientationValue for DeepWaterRouteCentreline does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.390565276S, 61.786044281E | DeepWaterRouteCentreline | basedOnFixedMarks false orientation/orientationValue 10 trafficFlow 4 |  | 20 |  | 1810:6873:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1296: An error “orientationValue for DeepWaterRouteCentreline does not correspond to the bearing of the line.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1055 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each NavigationalSystemOfMarks which OVERLAPS or is WITHIN another meta NavigationalSystemOfMarks. |  |  |  |  |  |  |  |  |  |
| Message | Overlapping NavigationalSystemOfMarks meta features. |  |  |  |  |  |  |  |  |  |
| Solution | Amend to remove overlaps |  |  |  | Conformity |  | PS 3.6.1 |  |  |  |
| Test Case No. 1 | Overlapping NavigationalSystemOfMarks meta features. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.407784957S, 61.821756341E | NavigationalSystemOfMarks |  |  | 1 |  | 1810:6869:1 |  |  | - |  |  |
|  | NavigationalSystemOfMarks |  |  | 4 |  | 1810:6864:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1055: An error “Overlapping NavigationalSystemOfMarks meta features.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1293 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each RecommendedRouteCentreline where trafficFlow is Equal to 1 (inbound) OR 2 (outbound) OR 3 (one-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value of orientationValue. |  |  |  |  |  |  |  |  |  |
| Message | orientationValue for RecommendedRouteCentreline does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Solution | Amend orientationValue to be consistent with orientation of the curve spatial feature it references. |  |  |  | Conformity |  | PS 15.9.1 |  |  |  |
| Test Case No. 1 | orientationValue for RecommendedRouteCentreline does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.348240578S, 61.769402829E | RecommendedRouteCentreline | basedOnFixedMarks True orientation/orientationValue 5 trafficFlow 1 |  | 10 |  | 1810:6857:1 |  |  | - |  |  |
| 32.348815566S, 61.769025493E | RecommendedRouteCentreline | basedOnFixedMarks True orientation/orientationValue 5 trafficFlow 2 |  | 16 |  | 1810:6865:1 |  |  |  |  |  |
| 32.349615159S, 61.768818856E | RecommendedRouteCentreline | basedOnFixedMarks True orientation/orientationValue 5 trafficFlow 3 |  | 19 |  | 1810:6872:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1293: An error “orientationValue for RecommendedRouteCentreline does not correspond to the bearing of the line.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1294 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each RecommendedRouteCentreline where orientationValue is Known AND trafficFlow is Equal to 4 (two-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value (or reciprocal value) of orientationValue. |  |  |  |  |  |  |  |  |  |
| Message | orientationValue for RecommendedRouteCentreline does not correspond to the bearing of the line |  |  |  |  |  |  |  |  |  |
| Solution | Amend orientationValue to be consistent with orientation of the curve spatial feature it references. |  |  |  | Conformity |  | PS 15.9.1 |  |  |  |
| Test Case No. 1 | orientationValue for RecommendedRouteCentreline does not correspond to the bearing of the line |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.395188916S, 61.786425291E | RecommendedRouteCentreline | basedOnFixedMarks False orientation/orientationValue 10 trafficFlow 4 |  | 27 |  | 1810:6881:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1294: An error “orientationValue for RecommendedRouteCentreline does not correspond to the bearing of the line.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1287 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each RecommendedTrack where orientationValue is Known AND trafficFlow is Equal to 4 (two-way) AND the bearing of the line is more than 5 degrees Greater than OR Less than the value (or reciprocal value) of orientationValue. |  |  |  |  |  |  |  |  |  |
| Message | orientationValue for RecommendedTrack does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Solution | Amend orientationValue to be consistent with orientation of the curve spatial feature it references. |  |  |  | Conformity |  | PS 15.5.1 |  |  |  |
| Test Case No. 1 | orientationValue for RecommendedTrack does not correspond to the bearing of the line. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.392795445S, 61.785890890E | RecommendedRouteCentreline | basedOnFixedMarks False orientation/orientationValue 10 trafficFlow 4 |  | 9 |  | 1810:6856:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1287: An error “orientationValue for RecommendedTrack does not correspond to the bearing of the line.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158005 |  | S‐158 Check |  | 101_1140 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Building, BuiltUpArea, Crane, FortifiedStructure, Landmark, SiloTank or WindTurbine without inTheWater set to True which OVERLAPS OR is COVERED_BY DepthArea, DredgedArea or an UnsurveyedArea. |  |  |  |  |  |  |  |  |  |
| Message | Feature situated within a DepthArea, DredgedArea or an UnsurveyedArea does not have boolean inTheWater set to TRUE |  |  |  |  |  |  |  |  |  |
| Solution | Set attribute inTheWater to true or amend data to ensure that the feature is situated over a suitable Skin of the Earth feature. |  |  |  | Conformity |  | PS 6.1, 6.2, 7.2, 7.3, 7.4, 7.5, 8.13 |  |  |  |
| Test Case No. 1 | Features without inTheWater set to TRUE |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.408342S, 61.800068E | Building Building Building BuiltUpArea BuiltUpArea BuiltUpArea Crane Crane Crane FortifiedStructure FortifiedStructure FortifiedStructure Landmark Landmark Landmark SiloTank SiloTank SiloTank |  |  | 56 58 60 59 61 66 65 70 74 64 69 73 48 68 72 62 67 71 |  | 1810:8738:1 1810:8725:1 1810:8732:1 1810:8726:1 1810:8739:1 1810:8733:1 1810:8734:1 1810:8740:1 1810:8727:1 1810:8735:1 1810:8741:1 1810:8728:1 1810:8736:1 1810:8742:1 1810:8729:1 1810:8737:1 1810:8743:1 1810:8730:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1140: An error “Feature situated within a DepthArea, DredgedArea or an UnsurveyedArea does not have boolean inTheWater set to TRUE” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | 6 |  |  |  |  |  |  |  |  |  |