- Test Dataset: 10100AA158006

S‐158 Recommended ENC Validation Checks covered in this section:

| S‐158 Check | Description |
| --- | --- |
| 101_1240 | For each Wreck OR Obstruction OR UnderwaterAwashRock where valueOfSounding is Unknown AND defaultClearanceDepth is Unknown. |
| 101_1254 | For each Obstruction feature of geometric primitive curve where categoryOfObstruction Equal to 23 (mangrove). |
| 101_1408 | For each Chart1Feature within the dataset. |
| 101_1268 | For each Obstruction OR Wreck where valueOfSounding is Present AND height is Present. |
| 101_1252 | For each Wreck where valueOfSounding is not Present AND categoryOfWreck is not Present. |

Secondary Errors

Critical – 1 (101_1005)

Error – None

Warnings – None

| Dataset Name | 10100AA158006 |  | S‐158 Check |  | 101_1240 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Wreck OR Obstruction OR UnderwaterAwashRock where valueOfSounding is Unknown AND defaultClearanceDepth is Unknown. |  |  |  |  |  |  |  |  |  |
| Message | Feature where attributes valueOfSounding and defaultClearanceDepth are Unknown. |  |  |  |  |  |  |  |  |  |
| Solution | Populate valueOfSounding or defaultClearanceDepth |  |  |  | Conformity |  | PS 13.4, 13.5, 13.6 |  |  |  |
| Test Case No. 1 | valueOfSounding and defaultClearanceDepth are not populated on an Obstruction. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.4974355S, 61.5084290E | Obstruction | categoryOfObstruction = 23 valueOfSounding = (Unknown) waterLevelEffect = 4 defaultClearanceDepth = (Unknown) surroundingDepth = 0 m |  | 74 |  | 1810:8678:1 |  |  | - |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1240: An error “Feature where attributes valueOfSounding and defaultClearanceDepth are Unknown.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158006 |  | S‐158 Check |  | 101_1254 |  |  | Type |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Obstruction feature of geometric primitive curve where categoryOfObstruction Equal to 23 (mangrove). |  |  |  |  |  |  |  |  |  |  |
| Message | Obstructions features of geometric primitive curve where categoryOfObstruction = 23(mangrove) is prohibited. |  |  |  |  |  |  |  |  |  |  |
| Solution | Encode as Obstruction of geometric primitive surface covered by intertidal area; or encode the seaward edge of the mangrove as Coastline bounding LandArea. |  |  |  | Conformity |  | PS 13.6.1.1 |  |  |  |  |
| Test Case No. 1 | Obstructions features of geometric primitive curve where categoryOfObstruction = 23(mangrove) is prohibited. |  |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| 32.4974355S, 61.5084290E | Obstruction | categoryOfObstruction = 23 valueOfSounding = (Unknown) waterLevelEffect = 4 defaultClearanceDepth = (Unknown) surroundingDepth = 0 m |  | 74 |  | 1810:8678:1 |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1254: An error “Obstructions features of geometric primitive curve where categoryOfObstruction = 23(mangrove) is prohibited.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158006 |  | S‐158 Check |  | 101_1408 |  |  | Type |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Chart1Feature within the dataset. |  |  |  |  |  |  |  |  |  |  |
| Message | Chart1Feature exists within the dataset but are prohibited for an ENC. |  |  |  |  |  |  |  |  |  |  |
| Solution | Remove Chart1Feature |  |  |  | Conformity |  | PS 32.1.1 |  |  |  |  |
| Test Case No. 1 | Prohibited Chart1Feature |  |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| 32.4995664S, 61.4959934E | Chart1Feature |  |  | 65 |  | 1810:8569:1 |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1408: An error “Chart1Feature exists within the dataset but are prohibited for an ENC.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158006 |  | S‐158 Check |  | 101_1268 |  |  | Type |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Obstruction OR Wreck where valueOfSounding is Present AND height is Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Obstruction or Wreck feature where valueOfSounding and height is Present. |  |  |  |  |  |  |  |  |  |  |
| Solution | Remove the value for either valueOfSounding or height |  |  |  | Conformity |  | PS 13.6 |  |  |  |  |
| Test Case No. 1 | Obstruction or Wreck feature where valueOfSounding and height is Present. |  |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| 32.4875919S, 61.4906630E | Wreck | height = 0.3 m valueOfSounding = 1.2 m waterLevelEffect = 1 surroundingDepth = 0.0 m |  | 72 |  | 1810:8668:1 |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1268: An error “Obstruction or Wreck feature where valueOfSounding and height is Present.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158006 |  | S‐158 Check |  | 101_1252 |  |  | Type |  | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Wreck where valueOfSounding is not Present AND categoryOfWreck is not Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Wreck feature where neither valueOfSounding or categoryOfWreck is Present |  |  |  |  |  |  |  |  |  |  |
| Solution | Populate either valueOfSounding or categoryOfWreck. |  |  |  | Conformity |  | PS 13.5 |  |  |  |  |
| Test Case No. 1 | Wreck feature where neither valueOfSounding or categoryOfWreck is Present. |  |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| 32.4902697S, 61.4915452E | Wreck | categoryOfWreck = UNDEFINED valueOfSounding = UNDEFINED waterLevelEffect = 1 surroundingDepth = 0.0 m |  | 73 |  | 1810:8679:1 |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1252: An error “Wreck feature where neither valueOfSounding or categoryOfWreck is Present” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | 101_1005 |  |  |  |  |  |  |  |  |  |  |