- Test Dataset: 10100AA158009

S‐158 Recommended ENC Validation Checks covered in this section:

| S‐158 Check | Description |
| --- | --- |
| 101_1013 | For each equipment feature which has a StructureEquipment feature association relationship AND where fixedDateRange.dateEnd or periodicDateRange.dateEnd attributes are Known AND the values of fixedDateRange.dateEnd or periodicDateRange.dateEnd are Greater than the values of fixedDateRange.dateEnd or periodicDateRange.dateEnd encoded on the structure feature. |
| 101_1012 | For each feature which has a StructureEquipment feature association relationship AND where fixedDateRange.dateStart or periodicDateRange.dateStart attributes are Known AND the values of fixedDateRange.dateStart or periodicDateRange.dateStart are Less than the values of fixedDateRange.dateStart or periodicDateRange.dateStart encoded on the structure feature. |
| 101_1016 | For each equipment feature where fixedDateRange.dateEnd is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where fixedDateRange.dateEnd is Known on the structure feature. |
| 101_1017 | For each equipment feature where periodicDateRange.dateEnd is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where periodicDateRange.dateEnd is Known on the structure feature. |
| 101_1015 | For each equipment feature where periodicDateRange.dateStart is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where periodicDateRange.dateStart is Known on the structure feature. |
| 101_1014 | For each equipment feature where fixedDateRange.dateStart is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where fixedDateRange.dateStart is Known on the structure feature. |
| 101_1026 | For each DataCoverage where the value of minimumDisplayScale is Not equal to other minimumDisplayScale values from other DataCoverage features. |
| 101_1005 | If any conditional mandatory attributes are not Present. |
| 101_1060 | For each Wreck OR UnderwaterAwashRock OR Obstruction where valueOfSounding is Unknown AND SpatialQuality.verticalUncertainty.uncertaintyFixed is Known. |
| 101_0022 | For each picture file which is not in the TIF format. |

Secondary Errors

Critical – None

Error – 5

Warnings – None

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1013 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each equipment feature which has a StructureEquipment feature association relationship AND where fixedDateRange.dateEnd or periodicDateRange.dateEnd attributes are Known AND the values of fixedDateRange.dateEnd or periodicDateRange.dateEnd are Greater than the values of fixedDateRange.dateEnd or periodicDateRange.dateEnd encoded on the structure feature. |  |  |  |  |  |  |  |  |  |
| Message | Temporal attributes on a equipment feature extend beyond those on the structure feature. |  |  |  |  |  |  |  |  |  |
| Solution | Amend temporal attributes to ensure that the date range of the equipment feature is equal to or within the date range of the structure feature. |  |  | Conformity |  |  | PS 2.4.8 |  |  |  |
| Test Case No. 1 | Temporal attributes on a equipment feature extend beyond those on the structure feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.452335700S, 61.703364900E | LightAirObstruction | colour 3 fixedDateRange/dateEnd 20120630 fixedDateRange/dateStart 20120624 rhythmOfLight 2,(1),4 sec, lightCharacteristic 2 (Flashing) signalGroup 1 (1) signalPeriod 4 sec status 5 |  |  | 12 | 1810:8834:1 |  |  | - |  |  |
| 32.452335700S, 61.703364900E | OffshorePlatform | colour 1 1 fixedDateRange/dateEnd 20120629 fixedDateRange/dateStart 20120623 |  |  | 25 | 1810:8848:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1013: An error “Temporal attributes on a equipment feature extend beyond those on the structure feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1012 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each feature which has a StructureEquipment feature association relationship AND where fixedDateRange.dateStart or periodicDateRange.dateStart attributes are Known AND the values of fixedDateRange.dateStart or periodicDateRange.dateStart are Less than the values of fixedDateRange.dateStart or periodicDateRange.dateStart encoded on the structure feature. |  |  |  |  |  |  |  |  |  |
| Message | Temporal attributes on a equipment feature extend beyond those on the structure feature. |  |  |  |  |  |  |  |  |  |
| Solution | Amend temporal attributes to ensure that the date range of the equipment feature is equal to or within the date range of the structure feature. |  |  | Conformity |  |  | PS 2.4.8 |  |  |  |
| Test Case No. 1 | Temporal attributes on a equipment feature extend beyond those on the structure feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.439262900S, 61.703886500E | LightAirObstruction | colour 3 fixedDateRange/dateEnd 20291231 fixedDateRange/dateStart 20231015 rhythmOfLight 2,(1),4 sec, lightCharacteristic 2 signalGroup 1 (1) signalPeriod 4 sec |  |  | 9 | 1810:8840:1 |  |  | - |  |  |
| 32.439262900S, 61.703886500E | OffshorePlatform | colour 1 fixedDateRange/dateEnd 20301231 fixedDateRange/dateStart 20240101 |  |  | 24 | 1810:8850:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1012: An error “Temporal attributes on a equipment feature extend beyond those on the structure feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1016 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each equipment feature where fixedDateRange.dateEnd is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where fixedDateRange.dateEnd is Known on the structure feature. |  |  |  |  |  |  |  |  |  |
| Message | fixedDateRange.dateEnd is not encoded for an equipment feature in an StructureEquipment association where fixedDateRange.dateEnd is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Solution | Populate fixedDateRange.dateEnd on the equipment to match the fixedDateRane.dateEnd on the structure feature. |  |  | Conformity |  |  | PS 2.4.8 |  |  |  |
| Test Case No. 1 | fixedDateRange.dateEnd is not encoded for an equipment feature in an StructureEquipment association where fixedDateRange.dateEnd is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.445181500S, 61.710039200E | LightAirObstruction | colour 3 fixedDateRange/ dateStart 20120623 rhythmOfLight 2,(1),4sec, lightCharacteristic 2 signalGroup (1) signalPeriod 4 sec |  |  | 13 | 1810:8830:1 |  |  | - |  |  |
| 32.445181500S, 61.710039200E | OffshorePlatform | colour 1 fixedDateRange/ dateEnd 20120624 fixedDateRange/dateStart 20120623 |  |  | 26 | 1810:8849:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1016: An error “fixedDateRange.dateEnd is not encoded for an equipment feature in an StructureEquipment association where fixedDateRange.dateEnd is encoded for the structure feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1017 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each equipment feature where periodicDateRange.dateEnd is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where periodicDateRange.dateEnd is Known on the structure feature. |  |  |  |  |  |  |  |  |  |
| Message | periodicDateRange.dateEnd is not encoded for a equipment feature in an StructureEquipment association where periodicDateRange.dateEnd is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Solution | Populate fixedDateRange.dateEnd on the equipment to match the fixedDateRane.dateEnd on the structure feature. |  |  | Conformity |  |  | PS 2.4.8 |  |  |  |
| Test Case No. 1 | periodicDateRange.dateEnd is not encoded for a equipment feature in an StructureEquipment association where periodicDateRange.dateEnd is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.445181500S, 61.710039200E | LightAllAround | colour 3 periodicDateRange/dateStart ----0401 rhythmOfLight 2 (Flashing),(1),4 sec, lightCharacteristic 2 signalGroup 1 (1) signalPeriod 4 sec status 1 5 |  |  | 14 | 1810:8854:1 |  |  | - |  |  |
| 32.445181500S, 61.710039200E | Daymark | colour 1 periodicDateRange/dateEnd ----1031 periodicDateRange/dateStart ----0401 topmarkDaymarkShape 1 |  |  | 27 | 1810:8855:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1017: An error “periodicDateRange.dateEnd is not encoded for a equipment feature in an StructureEquipment association where periodicDateRange.dateEnd is encoded for the structure feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1015 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each equipment feature where periodicDateRange.dateStart is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where periodicDateRange.dateStart is Known on the structure feature. |  |  |  |  |  |  |  |  |  |
| Message | periodicDateRange.dateStart is not encoded for an equipment feature in an StructureEquipment association where periodicDateRange.dateStart is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Solution | Populate periodicDateRange.dateStart on the equipment to match the periodicDateRange.dateStart on the structure feature. |  |  | Conformity |  |  | PS 2.4.8 |  |  |  |
| Test Case No. 1 | periodicDateRange.dateEnd is not encoded for a equipment feature in an StructureEquipment association where periodicDateRange.dateEnd is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.445181500S, 61.710039200E | LightAllAround | colour 3 periodicDateRange/datEnd ----1031 rhythmOfLight 2 (Flashing),(1),4 sec, lightCharacteristic 2 signalGroup (1) signalPeriod 4 sec status 5 |  |  | 15 | 1810:8856:1 |  |  | - |  |  |
| 32.445181500S, 61.710039200E | Daymark | colour 1 periodicDateRange/dateEnd ----1031 periodicDateRange/dateStart ----0401 topmarkDaymarkShape 1 |  |  | 28 | 1810:8857:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1015: An error “Populate periodicDateRange.dateStart on the equipment to match the periodicDateRange.dateStart on the structure feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1014 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each equipment feature where fixedDateRange.dateStart is Not Present or Unknown AND has a StructureEquipment feature association relationship AND where fixedDateRange.dateStart is Known on the structure feature. |  |  |  |  |  |  |  |  |  |
| Message | fixedDateRange.dateStart is not encoded for a equipment feature in an StructureEquipment association where fixedDateRange.dateStart is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Solution | Populate fixedDateRange.dateStart on the equipment to match the fixedDateRange.dateStart on the structure feature. |  |  | Conformity |  |  | PS 2.4.8 |  |  |  |
| Test Case No. 1 | fixedDateRange.dateStart is not encoded for a equipment feature in an StructureEquipment association where fixedDateRange.dateStart is encoded for the structure feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.445356000S, 61.697039600E | LightAirObstruction | colour 3 periodicDateRange/dateEnd ----1031 rhythmOfLight 2 (Flashing),(1),4 sec, lightCharacteristic 2 signalGroup 1 (1) signalPeriod 4 sec status 1 5 |  |  | 8 | 1810:8831:1 |  |  | - |  |  |
| 32.445181500S, 61.710039200E | OffshorePlatform | colour 1 3 (Red) fixedDateRange/dateEnd 20120624, rhythmOfLight 2 ,(1),4 sec, lightCharacteristic 2 signalGroup 1 (1) signalPeriod 4 sec |  |  | 23 | 1810:8846:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1014: An error “fixedDateRange.dateStart is not encoded for a equipment feature in an StructureEquipment association where fixedDateRange.dateStart is encoded for the structure feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_1026 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each DataCoverage where the value of minimumDisplayScale is Not equal to other minimumDisplayScale values from other DataCoverage features. |  |  |  |  |  |  |  |  |  |
| Message | DataCoverage meta features have different values of the attribute minimumDisplayScale. |  |  |  |  |  |  |  |  |  |
| Solution | Amend the minimumDisplayScale attribute values accordingly. |  |  | Conformity |  |  | PS 2.5.5 & 3.5.1 |  |  |  |
| Test Case No. 1 | DataCoverage meta features have different values of the attribute minimumDisplayScale. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.488983511S, 61.749273347E | DataCoverage | maximumDisplayScale 12000 minimumDisplayScale 90000 optimumDisplayScale 45000 |  |  | 4 | 1810:8858:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1026: An error “DataCoverage meta features have different values of the attribute minimumDisplayScale.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check |  | 101_1005 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If any conditional mandatory attributes are not Present. |  |  |  |  |  |  |  |  |  |
| Message | Missing conditional mandatory attribute. |  |  |  |  |  |  |  |  |  |
| Solution | Add missing attribute values |  |  |  | Conformity |  | PS 2.4.3 |  |  |  |
| Test Case No. 1 | Missing conditional mandatory attribute. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.476756000S, 61.685046000E | Obstruction | surroundingDepth 10 m waterLevelEffect 3 |  | 17 |  | 1810:8867:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1005: An error “Missing conditional mandatory attribute.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check |  | 101_1060 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each Wreck OR UnderwaterAwashRock OR Obstruction where valueOfSounding is Unknown AND SpatialQuality.verticalUncertainty.uncertaintyFixed is Known. |  |  |  |  |  |  |  |  |  |
| Message | Feature where valueOfSounding is unknown value for SpatialQuality.spatialAccuracy.verticalUncertainty.uncertaintyFixed. |  |  |  |  |  |  |  |  |  |
| Solution | Populate uncertaintyFixed with a value for this feature. |  |  |  | Conformity |  | PS 3.8.1.3, 13.4.1, 13.5.1, 13.6.1, 24.5.1 |  |  |  |
| Test Case No. 1 | Missing conditional mandatory attribute. |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |  |
| 32.484528700S, 61.696782800E | Obstruction | defaultClearanceDepth 0.2 m surroundingDepth 10 m valueOfSounding (Unknown) waterLevelEffect 3 |  | 20 |  | 1810:8864:1 |  |  |  |  |  |
| 32.484528700S, 61.691188000E | UnderwaterAwashRock | defaultClearanceDepth 0.2 m surroundingDepth 10 m valueOfSounding (Unknown) waterLevelEffect 3 |  | 19 |  | 1810:8863:1 |  |  |  |  |  |
| 32.484404000S, 61.685515300E | Wreck | defaultClearanceDepth 0.2 m surroundingDepth 10 m valueOfSounding (Unknown) waterLevelEffect 3 |  | 18 |  | 1810:8860:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_1060: An error “Feature where valueOfSounding is unknown value for SpatialQuality.spatialAccuracy.verticalUncertainty.uncertaintyFixed.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | 101_1238, 101_1250, 101_1265 |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158009 |  | S‐158 Check | 101_0022 |  |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each picture file which is not in the TIF format. |  |  |  |  |  |  |  |  |  |
| Message | Invalid picture file format |  |  |  |  |  |  |  |  |  |
| Solution | Ensure referenced picture files are in TIFF (6.0) format. |  |  | Conformity |  |  | PS 11.4 |  |  |  |
| Test Case No. 1 | Invalid picture file format |  |  |  |  |  |  |  |  |  |
| Location | Feature | Attributes |  |  | FRID | FOID |  |  | VRID |  |  |
| 32.484404000S, 61.685515300E | Daymark | colour 1 pictorialRepresentation 10100AA_DAYMARK.gif topmarkDaymarkShape 1 |  |  | 16 | 1810:8859:1 |  |  |  |  |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_0022: An error “Invalid picture file format.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | 100_0006 |  |  |  |  |  |  |  |  |  |