- Test Dataset: 10100AA158011

S‐101 Recommended ENC Validation Checks covered in this section:

| S‐101 Check | Description |
| --- | --- |
| 101_2007 | For each COCC field where COCC-COUI is Equal to 1 (Insert) or 3 (Modify) AND the value of COCC-NCOR is Not equal to the number of coordinates referenced contained within the C2IL field. |
| 101_2008 | For each COCC field where CCOC-COUI is Equal to 2 (Delete) where the C2IL field is Present. |
| 101_2009 | For each COCC field where CCOC-COIX where the index position is invalid. |
| 101_2010 | For each CCID field where CCID-RUIN is Equal to 1 (Insert) where the CUCO field is not Present. |
| 101_2011 | For each CCID field where CCID-RUIN is Equal to 2 (Delete) where the CUCO field is Present. |
| 101_2012 | For each CCID field where CCID-RUIN is Equal to 3 (Modify) where the CCOC field is not Present. |
| 101_2013 | For each CCID field where CCID-RUIN is Equal to 1 (Insert) or 2 (Delete) where the CCOC field is Present. |
| 101_2014 | For each CCOC field where CCOC-CCUI is Equal to 2 (Delete) where the CUCO field is Present. |
| 101_2015 | For each CCOC field where CCOC-CCUI is Equal to 1 (Insert) or 3 (Modify) where the value of CCOC-NCCO is Not equal to the number of records referenced by CUCO-RRNM. |
| 101_2016 | For each CCOC-CCIX subfield where the index position for updating is invalid. |
| 101_2020 | If the order of the data in a base or update file is not correct, except for when: 1. Isolated nodes are listed before isolated nodes OR 2. Connected nodes are listed before isolated nodes OR 3. Connected nodes are listed before isolated nodes OR 4. Geo features are listed before Meta features. |

Secondary Errors

Critical – None

Error – None

Warnings – None

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2007 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each COCC field where COCC-COUI is Equal to 1 (Insert) or 3 (Modify) AND the value of COCC-NCOR is Not equal to the number of coordinates referenced contained within the C2IL field. |  |  |  |  |  |  |  |  |  |  |
| Message | COCC.NCOR is not equal to the number of cooordinates in the C2IL field. |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend CCOC.NCOR to refect the corrEct number of referenced coodinates |  |  |  |  | Conformity |  | PS B-6.1.18 |  |  |  |
| Test Case No. 1 | COCC.NCOR is not equal to the number of cooordinates in the C2IL field. |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2007: An error “COCC.NCOR is not equal to the number of cooordinates in the C2IL field.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2008 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each COCC field where CCOC-COUI is Equal to 2 (Delete) where the C2IL field is Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Invalid C2IL field |  |  |  |  |  |  |  |  |  |  |
| Solution | Remove C2IL field |  |  |  |  | Conformity |  | PS B-6.1.18 |  |  |  |
| Test Case No. 1 | Invalid C2IL field |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2008: An error “Invalid C2IL field.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2009 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each COCC field where CCOC-COIX where the index position is invalid. |  |  |  |  |  |  |  |  |  |  |
| Message | Invalid value of COIX subfield |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend CCOC-COIX |  |  |  |  | Conformity |  | PS B-6.1.18 |  |  |  |
| Test Case No. 1 | Invalid value of COIX subfield |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2009: An error “Invalid value of COIX subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2010 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCID field where CCID-RUIN is Equal to 1 (Insert) where the CUCO field is not Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Missing CUCC subfield |  |  |  |  |  |  |  |  |  |  |
| Solution | Add CUCC subfield |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | Missing CUCC subfield |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2010: An error “Missing CUCC subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2011 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCID field where CCID-RUIN is Equal to 2 (Delete) where the CUCO field is Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Invalid CUCO subfield |  |  |  |  |  |  |  |  |  |  |
| Solution | Remove CUCO subfield |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | Invalid CUCO subfield |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2010: An error “Invalid CUCO subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2012 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCID field where CCID-RUIN is Equal to 3 (Modify) where the CCOC field is not Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Missing CCOC subfield |  |  |  |  |  |  |  |  |  |  |
| Solution | Add CCOC subfield |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | Missing CCOC subfield |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2012: An error “Missing CCOC subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2013 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCID field where CCID-RUIN is Equal to 1 (Insert) or 2 (Delete) where the CCOC field is Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Invalid CCOC subfield |  |  |  |  |  |  |  |  |  |  |
| Solution | Remove CCOC subfield |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | Invalid CCOC subfield |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2013: An error “Invalid CCOC subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2014 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCOC field where CCOC-CCUI is Equal to 2 (Delete) where the CUCO field is Present. |  |  |  |  |  |  |  |  |  |  |
| Message | Invalid CUCO subfield |  |  |  |  |  |  |  |  |  |  |
| Solution | Remove CUCO subfield |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | Invalid CUCO subfield |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2014: An error “Invalid CUCO subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2015 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCOC field where CCOC-CCUI is Equal to 1 (Insert) or 3 (Modify) where the value of CCOC-NCCO is Not equal to the number of records referenced by CUCO-RRNM. |  |  |  |  |  |  |  |  |  |  |
| Message | CCOC.NCCO is not equal to the number of referenced CUCO records. |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend CCOC.NCCO to refect the corrEct number of referenced CUCO records |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | CCOC.NCCO is not equal to the number of referenced CUCO records. |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2015: An error “CCOC.NCCO is not equal to the number of referenced CUCO records.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2016 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For each CCOC-CCIX subfield where the index position for updating is invalid. |  |  |  |  |  |  |  |  |  |  |
| Message | Invalid index position |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend CCOC.CCIX |  |  |  |  | Conformity |  | PS B-6.1.25 |  |  |  |
| Test Case No. 1 | Invalid index position |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2016: An error “Invalid index position.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158011 |  |  | S‐158 Check |  | 101_2020 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If the order of the data in a base or update file is not correct, except for when: 1. Isolated nodes are listed before isolated nodes OR 2. Connected nodes are listed before isolated nodes OR 3. Connected nodes are listed before isolated nodes OR 4. Geo features are listed before Meta features. |  |  |  |  |  |  |  |  |  |  |
| Message | Incorrect order of data records |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend order of data records. |  |  |  |  | Conformity |  | PS B-1 |  |  |  |
| Test Case No. 1 | Incorrect order of data records |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2020: An error “Incorrect order of data records.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |