- Test Dataset: 10100AA158012

S‐101 Recommended ENC Validation Checks covered in this section:

| S‐101 Check | Description |
| --- | --- |
| 101_0006 | If the limits of an update cell file are not identical to the limits of the base cell to which they apply. |
| 101_0008 | For a base cell file, if the limits contained in the subfields boundingBox and boundingPolygon of the S-100_DataCoverage field of the S-100 Exchange Set are Not equal to the furthest coordinates of the DataCoverage in the corresponding base cell file. |
| 101_0020 | If the fourth to seventh characters of the dataset file name do not correspond to the value of the FOID-AGEN subfield. |
| 101_0021 | If the data set is not a re-issue AND the updateNumber subfield is not equivalent to the extension of the data set file name. |
| 101_2005 | If the value of the DSID-DSRD subfield of the update dataset file is Less than the value on the DSID-DSRD subfield of the base dataset file. |

Secondary Errors

Critical – None

Error – None

Warnings – None

| Dataset Name | 10100AA158012 |  |  | S‐158 Check |  | 101_0006 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If the limits of an update cell file are not identical to the limits of the base cell to which they apply. |  |  |  |  |  |  |  |  |  |  |
| Message | Cell extents of update file differ from that of the parent base file |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend limits of update file or create a New edition if the extents of the cell are to be amended. |  |  |  |  | Conformity |  | PS 4.5.2 |  |  |  |
| Test Case No. 1 | Cell extents of update file differ from that of the parent base file |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_0006: An error “Cell extents of update file differ from that of the parent base file.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158012 |  |  | S‐158 Check |  | 101_0008 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | For a base cell file, if the limits contained in the subfields boundingBox and boundingPolygon of the S-100_DataCoverage field of the S-100 Exchange Set are Not equal to the furthest coordinates of the DataCoverage in the corresponding base cell file. |  |  |  |  |  |  |  |  |  |  |
| Message | Cell extents differ from that given in S100_DatasetDiscoveryMetadata |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend limits in S100_DatasetDiscoveryMetadata or DataCoverage extents to agree. |  |  |  |  | Conformity |  | PS 4.5.2 |  |  |  |
| Test Case No. 1 | Cell extents differ from that given in S100_DatasetDiscoveryMetadata |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_0008: An error “Cell extents differ from that given in S100_DatasetDiscoveryMetadata.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158012 |  |  | S‐158 Check |  | 101_0020 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If the fourth to seventh characters of the dataset file name do not correspond to the value of the FOID-AGEN subfield. |  |  |  |  |  |  |  |  |  |  |
| Message | Data set file name does not include the agency code corresponding to that set in the AGEN subfield of the DSID field. |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend limits in S100_DatasetDiscoveryMetadata or DataCoverage extents to agree. |  |  |  |  | Conformity |  | PS 4.5.2 |  |  |  |
| Test Case No. 1 | Data set file name does not include the agency code corresponding to that set in the AGEN subfield of the DSID field. |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_0020: An error “Data set file name does not include the agency code corresponding to that set in the AGEN subfield of the DSID field.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158012 |  |  | S‐158 Check |  | 101_0021 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If the data set is not a re-issue AND the updateNumber subfield is not equivalent to the extension of the data set file name. |  |  |  |  |  |  |  |  |  |  |
| Message | updateNumber is incorrect or not equivalent to the update dataset file name extension. |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend updateNumber |  |  |  |  | Conformity |  | PS 11.3.3, 12.1.2 |  |  |  |
| Test Case No. 1 | updateNumber is incorrect or not equivalent to the update dataset file name extension. |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_0021: An error “updateNumber is incorrect or not equivalent to the update dataset file name extension.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |

| Dataset Name | 10100AA158012 |  |  | S‐158 Check |  | 101_2005 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description | If the value of the DSID-DSRD subfield of the update dataset file is Less than the value on the DSID-DSRD subfield of the base dataset file. |  |  |  |  |  |  |  |  |  |  |
| Message | Update file dataset reference date precedes the base file dataset reference date. |  |  |  |  |  |  |  |  |  |  |
| Solution | Amend updateNumber |  |  |  |  | Conformity |  | PS B-6.1.1 |  |  |  |
| Test Case No. 1 | Update file dataset reference date precedes the base file dataset reference date. |  |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| - |  | - | - |  | - |  | - |  |  | - |  |
| Screen Capture | - |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101_2005: An error “Update file dataset reference date precedes the base file dataset reference date.” must be triggered. |  |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |  |