- Test Dataset: 10100AA158007

S‐158 Recommended ENC Validation Checks covered in this section:

| S‐158 Check | Description |
| --- | --- |
| 101_1317 | For each feature of geometric primitive point, forming the same navigational aid, which does not reference the same spatial object. |
| 101_2017 | For each FRID field where FRID-RUIN is Equal to 1 (Insert) where the FOID field is not Present. |
| 101_2018 | For each FRID field where FRID-RUIN is Equal to 2 (Delete) or 3 (Modify) where the FOID field is Present. |

Secondary Errors

Critical – None

Error – None

Warnings – None

| Dataset Name |  | 10100AA158007 |  | S‐158 Check |  | 101_1317 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description |  | For each feature of geometric primitive point, forming the same navigational aid, which does not reference the same spatial object. |  |  |  |  |  |  |  |  |  |
| Message |  | Components of a navigational aid feature do not reference the same spatial feature. |  |  |  |  |  |  |  |  |  |
| Solution |  | Ensure all components of a point navigational aid feature reference to the same spatial feature. |  |  |  | Conformity |  | PS 18.2 |  |  |  |
| Test Case No. 1 |  | Components of a navigational aid feature do not reference the same spatial feature. |  |  |  |  |  |  |  |  |  |
| Location | Feature |  | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results |  | 101_1317: An error “Components of a navigational aid feature do not reference the same spatial feature.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors |  | None |  |  |  |  |  |  |  |  |  |

| Dataset Name |  | 10100AA158007 Update: 1 |  | S‐158 Check |  | 101_2017 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description |  | For each FRID field where FRID-RUIN is Equal to 1 (Insert) where the FOID field is not Present. |  |  |  |  |  |  |  |  |  |
| Message |  | Missing FOID subfield |  |  |  |  |  |  |  |  |  |
| Solution |  | Add FOID |  |  |  | Conformity |  | PS B-6.1.29 |  |  |  |
| Test Case No. 1 |  | Missing FOID subfield |  |  |  |  |  |  |  |  |  |
| Location | Feature |  | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results |  | 101_2017: An error “Missing FOID subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors |  | None |  |  |  |  |  |  |  |  |  |

| Dataset Name |  | 10100AA158007 Update: 1 |  | S‐158 Check |  | 100_2018 |  |  | Type |  | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-158 Description |  | For each FRID field where FRID-RUIN is Equal to 2 (Delete) or 3 (Modify) where the FOID field is Present. |  |  |  |  |  |  |  |  |  |
| Message |  | Invalid FOID subfield |  |  |  |  |  |  |  |  |  |
| Solution |  | Remove FOID subfield |  |  |  | Conformity |  | PS B-6.1.29 |  |  |  |
| Test Case No. 1 |  | Invalid FOID subfield |  |  |  |  |  |  |  |  |  |
| Location | Feature |  | Attributes |  | FRID |  | FOID |  |  | VRID |  |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results |  | 100_2018: An error “Invalid FOID subfield.” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors |  | None |  |  |  |  |  |  |  |  |  |